BAY COUNTY LIBRARY SYSTEM

FALSE FIRE ALARM

ALICE AND JACK WIRT LIBRARY

When you are certain it is a false fire alarm

- Call – Solucient Security System 989-686-3194

Wirt System Number is: 6508   Wirt Code: 2706

- Call 911 and report a False Alarm

- Silence alarm by pressing the “Alarm Silence” & “Panel Tone Silence” button on the Fire Alarm Control Panel located: on the wall right of the staff elevator

- Use intercom to notify patrons that it is a false alarm

- Get alarm box key from key box by circ workroom computers along with the replacement tubes

- Reset the pulled alarm: unlock alarm from wall, remove broken pin, close pull, place new pin in, close alarm box & lock with key.  If it is a covered pull station, there is a cover alarm.  Press the button on inside of the top of cover or replace cover and that alarm goes off.

- Reset the system by pressing “System Reset” on the Fire Alarm Control Panel

- Clear the Solucient Security System by putting in the Wirt Code twice

2706 1               2706 1

- Call Maintenance Supervisor (989-225-4255)