BAY COUNTY LIBRARY SYSTEM                                                                      FALSE FIRE ALARM PROCEDURE                                                                  PINCONNING BRANCH LIBRARY

When you are certain it is a false fire alarm

1. Call 911 and report a False Alarm

2. Call - Solucient Alarm 989-686-3194 and report a false alarm

Branch System Number is: 6467 Branch Code: 0723

3. Silence alarm by pressing the “Alarm Silence” button on the Simplex Fire Alarm Control Panel in Electronics Room (next to the staff break room) This is the first panel on the left.

4. Notify patrons that it is a false alarm

5. Get alarm box key along with the replacement tubes from envelope taped to AED by page desk

6. Reset the pulled alarm – push the handle back into place and replace the tube

7. Reset the system by pressing “System Reset” on the Simplex Fire Alarm Control Panel

8. Clear the Solucient alarm by putting in the branch code twice: 07231

9. Call Scott (989-225-4255) to report a false alarm